ALARM 088 spindle stopped fault

A few possible reasons:

- Spindle ON/OFF Relay is not working correctly

  - Find Spindle Relay (will have FWD and DCM wires attached) (Usually 4th/5th from left)

    - Replace relay (can swap with unused vacuum pump (last relay))

- VFD has fault

  - Restart machine to clear VFD fault

  - IF fault does not clear, need to look up fault

- Input of “no fault” from VFD not being received by controller

  - Check input I/06 wire

    - Controller end

    - Terminal bridge end

    - Inside VFD (picture below) (+24V and I/06 on first relay [yellow])